# ZolaDOS

Revised: 24 Nov 2025

ZolaDOS is being run as a systemd service on imp and zd0. The unit file is: /etc/systemd/system/zolados.service.

To update the software, run goup on the dev machine. This won't upload because the target is set to 'local'.

On imp/zd0, run the getz script.

| Zolatron |  | RPi |  |  | RPi |  | Zolatron |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 1 | +3.3V |  | +5V | 2 |  |  |
|  |  | 3 | GPIO2 SDA |  | +5V | 4 |  |  |
|  |  | 5 | GPIO3 SCL |  | GND | 6 |  |  |
| D0 | PA0 | 7 | GPIO4 |  | TX | 8 | RX on DUART PORTA |  |
|  |  | 9 | GND |  | RX | 10 | TX on DUART PORTA |  |
| D1 | PA1 | 11 | GPIO17 |  | GPIO18 | 12 | PA2 | D2 |
| D3 | PA3 | 13 | GPIO27 |  | GND | 14 |  |  |
| D4 | PA4 | 15 | GPIO22 |  | GPIO23 | 16 | PA5 | D5 |
|  |  | 17 | +3.3V |  | GPIO24 | 18 | PA6 | D6 |
|  |  | 19 | GPIO10 |  | GND | 20 |  |  |
|  |  | 21 | GPIO9 |  | GPIO25 | 22 | PA7 | D7 |
|  |  | 23 | GPIO11 |  | GPIO8 | 24 |  | Ext. LED |
|  |  | 25 | GND |  | GPIO7 | 26 |  | /IRQ |
|  |  | 27 | X |  | X | 28 |  |  |
| /CA | PB0 | 29 | GPIO5 |  | GND | 30 |  |  |
| /CR | PB1 | 31 | GPIO6 |  | GPIO12 | 32 | PB2 | /CO |
|  |  | 33 | GPIO13 |  | GND | 34 |  |  |
| /SR | PB4 | 35 | GPIO19 |  | GPIO16 | 36 | PB5 | /SA |
| S4 | PB6 | 37 | GPIO26 |  | GPIO20 | 38 | PB7 | INTSEL |
|  |  | 39 | GND |  | GPIO21 | 40 |  | /RST |

Control Signals

| Client (Z64) |  | DIR |  | Server (RPi) |
| --- | --- | --- | --- | --- |
| Client Active – /CA Taken low by the Z64 to indicate that it wishes to initiate an action. It's held low by the Z64 until everything is done. When sending, taken high again to indicate that everything has been sent. | PB0 | --> | GPIO5 PIN 29 | Server polls this line. When it goes low, that initiates a Zolados process. |
| Client Ready – /CR When idle, the client wishes to initiate a read or write operation. When sending, a byte has been placed on the data bus. When receiving, the byte on the data bus has been received & processed. | PB1 | --> | GPIO6 PIN 31 | After /CA has gone low, server monitors this line. When it goes low, server strobes /SR low to acknowledge. |
| Client Online – /CO Normally pulled high, this is taken low by the Z64 to show that it is powered & connected. | PB2 | --> | GPIO13 PIN 33 | Server constantly polls this line to check that client is active & available. |
| Data Direction – DDIR Sets the direction of the data bus. Connects to pin 2 of the 74LVC4245. HIGH = Z64 to Server LOW = Server to Z64 | PB3 | – | — | (Connects to level shifter) |
| Polled during read/write operations. | PB4 | <-- | GPIO19 PIN 35 | Server Ready – /SR When sending, a byte has been placed on the data bus. When receiving, the byte on the data bus has been received & processed. |
| Polled during receive operations. | PB5 | <-- | GPIO16 PIN 36 | Server Active – /SA When sending, taken low to indicate the server is sending data. Goes high again when server has finished sending data. |
| Checked by Z64 when an interrupt occurs to see if it was the RPi that generated it. (Not currently used.) | PB6 | <-- | GPIO26 PIN 37 | INTSEL This is set high by the RPi before it generates an interrupt. |
| (reserved for future use) | PB7 | <-- | GPIO20 PIN 38 | S4 (reserved for future use) |
|  | /RST | <— | GPIO21 PIN 40 | /RST Reset Z64 |

LOAD/ SAVE PROCESS

LOAD functions

cmdprcLOAD

- Set FILE_ADDR to USR_START

- jsr zd_getfile

  - jsr read_filename

  - jsr zd_loadfile

    - jsr zd_handshake

      - jsr zd_init_process

        - jsr zd_signalDelay (x2)

        - jsr zd_waitForSR (starts, checks and stops timer)

      - jsr zd_send_strbuf

        - jsr zd_waitForSRoff (starts, checks and stops timer)

        - jsr zd_signalDelay

        - jsr zd_waitForSR

      - jsr zd_svr_resp

        - jsr zd_waitForSA (starts, checks and stops timer)

        - sr zd_waitForSR (starts, checks and stops timer)

    - jsr zd_waitForSAoff (starts, checks and stops timer)

    - jsr zd_rcv_data

      - jsr zd_waitForSA (starts, checks and stops timer)

      - loop:

        - jsr zd_waitForSR (starts, checks and stops timer)

        - jsr zd_waitForSRoff (starts, checks and stops timer)

        - jsr zd_strobeDelay (timerloop)

      - jsr zd_strobeDelay (timer loop)

      - jsr zd_waitForSAoff (starts, checks and stops timer)

cmdprcXLOAD

- set A to ZD_OPCODE_XLOAD

- jsr xload_file

  - Read filename from STDIN_BUF to STR_BUF

  - Check currently selected RAM bank is available

  - Set FILE_ADDR to EXTMEM_START

  - jsr OSZDLOAD (zd_loadfile)

cmdprcXOPEN

- set A to ZD_OPCODE_DLOAD

- jsr xload_file

  - Read filename from STDIN_BUF to STR_BUF

  - Check currently selected RAM bank is available

  - Set FILE_ADDR to EXTMEM_START

  - jsr OSZDLOAD (zd_loadfile)

SAVE functions

cmdprocDUMP

- Read hex address pair

- Read filename from STDIN_BUF to STR_BUF

- jsr OSZDSAVE (zd_save_data)

cmdprcSAVE

- Check executable code is currently loaded

- Read filename from STDIN_BUF to STR_BUF

- Set TEMP_ADDR_A to USR_START

- Set TEMP_ADDR_B to LOMEM

- jsr OSZDSAVE (zd_save_data)

Common functions

OSZDLOAD – funcs_ZolaDOS.asm :: .zd_loadfile

- zd_handshake

  - zd_init_process - send opcode, get response

  - zd_send_strbuf - send filename

  - zd_svr_resp

- zd_rcv_data – loads to address specified by FILE_ADDR/+1

OSZDSAVE – funcs_ZolaDOS.asm :: .zd_save_data

(Might replace first three steps with zd_handshake)

- zd_handshake

  - zd_init_process - send opcode, get response

  - zd_send_strbuf - send filename

  - zd_svr_resp

- zd_send_data

zd_send_data

FUNCTIONS

| Z64 -> SERVER (Z64 -> SERVER) |  |
| --- | --- |
| ZOLATRON | SERVER |

| INIT PROCESS (Z64 -> SERVER) |  |
| --- | --- |
| ZOLATRON | SERVER |
|  | (IDLE) - /SA and /SR inactive, DATA port = input |
| (.zd_init_process) |  |
| Set DDR to OUTPUT |  |
| Sets /CA - LOW | Watching for /CA to be ACTIVE |
|  | Watch for /CR |
| Load opcode on DATA port |  |
| Sets /CR - LOW |  |
| Wait for /SR to be LOW |  |
|  | Read DATA port for code |
|  | Strobe /SR |
| Set /CR - HIGH |  |
| Set /CA - HIGH |  |

| HANDSHAKE (Z64 -> SERVER) |  |
| --- | --- |
| ZOLATRON | SERVER |
| (.zd_handshake) |  |
| INIT PROCESS |  |
| SEND STRBUF |  |
| Set DDR to INPUT |  |

| SEND STRBUF (Z64 -> SERVER) |  |
| --- | --- |
| ZOLATRON | SERVER |
| (.zd_send_strbuf) |  |
|  | Wait for /CA |
| Set /CA - LOW |  |
|  | — GET TEXT loop |
|  | Wait for /CR - ACTIVE |
| Wait for /SR to be HIGH |  |
| Put byte on DATA port |  |
| Set /CR - LOW |  |
| Wait for /SR to be LOW |  |
|  | Read char from data port |
|  | Check if valid character |
|  | Strobe /SR |
| Set /CR - HIGH |  |

| FILE DATA Data from server to Z64 |  |
| --- | --- |
| ZOLATRON | SERVER |
| (.zd_recv_data) | Set /SR to INACTIVE |
| Wait for /SA to be ACTIVE |  |
|  | Get list of files |
|  | Set /SA to ACTIVE |
|  | Set DATA port to OUTPUT |
| — Loop for each file |  |
| — Loop for each filename |  |
|  | SENDBYTE (char) |
| — end filename loop |  |
|  | SENDBYTE (0) |
| — end file loop |  |
|  | SENDBYTE (DataEndCode = 255) |
|  | Set /SA to INACTIVE |

| SERVER SENDBYTE (SERVER -> Z64) |  |
| --- | --- |
| ZOLATRON | SERVER |
| Wait for /SR to be ACTIVE |  |
|  | Set DATA port value |
|  | Strobe /SR |
|  | Wait for /CR - ACTIVE |
| Read byte |  |
| Set /CR - LOW |  |
| Set /CR - HIGH |  |
|  | Wait for /CR - INACTIVE |

PROCESSES

After each process, set DATA port to INPUT.

## LOAD FILE

| ZOLATRON | SERVER |
| --- | --- |
| (.zd_handshake) |  |
|  | Load file |
| SEND RESPONSE CODE |  |
|  | Wait for /CA to be INACTIVE |
|  | Set DATA port to OUTPUT |
|  | Set /SA to ACTIVE |
|  | Wait for /CR |
|  | Set code on DATA port |
|  | Strobe /SR |
|  | Set /SA to INACTIVE |
|  | Delay - 100µs |
|  | Set /SA to ACTIVE |
|  | Loop while !EOF |
|  | Read next byte from file data |
| SEND BYTE |  |
|  | Set DATA port value |
|  | Strobe /SR |
|  | Wait for /CR - ACTIVE |
|  | Wait for /CR - INACTIVE |
|  | Set /SA to INACTIVE |

## LS

| ZOLATRON | SERVER |
| --- | --- |
| Load opcode on DATA port |  |
| INIT PROCESS |  |
| FILE DATA |  |

NOT SURE WHAT THIS IS

| Zolatron | RPi |
| --- | --- |
| sets CR to OFF | set SR to ON |
| sets data direction to OUTPUT | waiting for CA to go ON |
| sets CA to ON |  |
| waits for server SR to go OFF |  |
| DATA LOOP |  |
| — puts byte on data bus | waits for CR to go ON |
| — sets CR to ON |  |
| — strobe delay | reads byte |
| — sets CR to OFF |  |
| — waits for SR to go ON |  |
|  | strobes SR ON then OFF |
| — waits for SR to go OFF | waits for CR to go OFF erroring here |
| — increments address pointer |  |
| — loops back | checks CA == ON otherwise ends loop |
| sets CA to OFF |  |

File Headers

File type codes

These live at byte $03 in the header.

| CODE | TYPE | HEX | NOTE |
| --- | --- | --- | --- |
| D | Data | 44 | Pure data files. |
| E | Executable | 45 | Executable programs that run either at USR_START or EXTMEM_START. |
| O | Overlay | 4F | Code designed to be swapped in & out to complement a program in main memory. |
| X | OS extension | 58 | OS extensions designed to be run from the command line. |

Data files

These might be (eg) lookup tables.

| BYTE | CONTENTS | NOTE |
| --- | --- | --- |
| 0 | $FF |  |
| 1 | $00 or load address LSB | These bytes can be zero or the 16-bit address of the usual load location for the data. |
| 2 | $00 or load address MSB |  |
| 3 | D’, ‘E’, ‘O’ or ‘X’ | Type code |

Executable files

Programs designed to be run in main memory (at USR_START) or extended memory (at EXTMEM_START).

Most files – and especially executable files – should start with a header.

| BYTE | CONTENTS | NOTE |
| --- | --- | --- |
| 0 | $4C | JMP instruction - jmp .startprog |
| 1 | 16-bit address of .startprog label | Address for previous JMP instruction. Machine will jump over this header info. |
| 2 |  |  |
| 3 | E’ | Type code |
| 4 | 16-bit address of .header label | Also first byte of code, so will match either USR_PAGE or EXTMEM_LOC |
| 5 |  |  |
| 6 | 16-bit address of .reset label | Usually the same as the .startprog label, but not necessarily. Where to jump to when resetting. |
| 7 |  |  |
| 8 | 16-bit address of .endcode label | Address of first byte available after end of program |
| 9 |  |  |
| A | -- reserved -- | (For future expansion) |
| B | -- reserved -- | (For future expansion) |
| C | -- reserved -- | (For future expansion) |
| D | .prog_name | Nul-terminated string containing short version of program name |
| varies | .version_string | Nul-terminated version string |

Overlays

Not sure if/how these are going to work, so reserved for future expansion.

| BYTE | CONTENTS | NOTE |
| --- | --- | --- |
| 0 | $4C | Jump to ... something. |
| 1 | Address of something to jump to – probably some kind of table |  |
| 2 |  |  |
| 3 | O’ | Type code |

OS Extensions

-- WORK IN PROGRESS --

OS extension files are designed to contain additional OS features. They load into extended memory.

Each package will be limited to 8KB. The package should start with a header as usual. In this case, the header will include a command table that lists the commands provided by the package followed by the address of the related function.

.command_table

equs "FOO",0

equw foo_function

equs "BAR",0

equw bar_function

equb EOTBL_MKR

.foo_function

...

rts

.bar_function

...

rts

The OS can poll through the extended memory banks. In each one it:

- 1. Checks that the type code is 'X'. If not, go on to next bank.

- 2. Read the address of the command_table from bytes 2 & 3 of the header.

- 3. Reads the next string in the command_table and matches it against the entered command.

  - 4. If it matches, read the address bytes. Done.

  - 5. If it doesn't match:

    - 6. Skip two bytes (ie, skip over the address)

    - 7. Check to see if next byte is = EOTBL_MKR.

      - 8. If it is, done with this bank – go to next one.

      - 9. If it isn't, loop back to step 3.

The code should also contain a routine to print a list of available commands. This code starts at .startprog.

| BYTE | CONTENTS | NOTE |
| --- | --- | --- |
| 0 | $4C | JMP instruction - jmp .startprog |
| 1 | 16-bit address of .startprog label | Address for previous JMP instruction. Machine will jump over this header info. |
| 2 |  |  |
| 3 | X' | Type code |
| 4 | 16-bit address of .command_table label |  |
| 5 |  |  |
| 6 | -- reserved -- |  |
| 7 | -- reserved -- |  |
| 8 | -- reserved -- |  |
| 9 | -- reserved -- |  |
| A | .package_name | Nul-terminated string containing short version of package name |
| varies | .version_string | Nul-terminated version string |

OPCODES & CONSTANTS

The actual numerical value of these may change, so here we'll just list the constant names used.

These are defined in:

- 6502 code: cfg_ZolaDOS.asm

- Go code: zdlib/zdconsts.go

The names given are for the ZolaDOS (Go) code, but the 6502 constants are the same, but prefixed with ‘ZD_’.

| NAME | Code | Function |
| --- | --- | --- |
| OPCODE_LOAD | 2 - 0x02 0000 0010 | LOAD - load executable .EXE file |
| OPCODE_DLOAD | 3 - 0x03 0000 0011 | DATA LOAD - load data files - no ext added |
| OPCODE_XLOAD | 4 - 0x04 0000 0100 | LOAD executable into extended memory - ROM images |
| OPCODE_LS | 8 - 0x08 0000 1000 | List storage |
| OPCODE_OPENR | 10 - 0x0A 0000 1010 | OPEN file for reading |
| OPCODE_OPENW | 11 - 0x0B 0000 1011 | OPEN file for writing |
| OPCODE_CLOSE | 12 - 0x0C 0000 1100 | CLOSE file |
| OPCODE_RBLK | 30 - 0x1E 0001 1110 | Read block |
| OPCODE_RBYTE | 31 - 0x1F 0001 1111 | Read byte |
| OPCODE_RSTR | 32 - 0x20 0010 0000 | Read null-terminated string |
| OPCODE_WBLK | 40 - 0x28 0010 1000 | Write block |
| OPCODE_WBYTE | 41 - 0x29 0010 1001 | Write byte |
| OPCODE_WSTR | 42 - 0x2A 0010 1010 | Write null-terminated string |
| FILE OPERATIONS |  |  |
| OPCODE_DEL | 72 - 0x48 0111 0010 | Delete file |
| OPCODE_MV | 96 - 0x60 1001 0110 | Move (rename) file |
| BINARY DUMPS: Following will cause ZolaDOS to append '.BIN' to the filename. |  |  |
| OPCODE_DUMP_CRT | 128 - 0x80 1000 0000 | Save - create (no overwrite) |
| OPCODE_DUMP_OVR | 129 - 0x81 1000 0001 | Save - create (overwrite) |
| OPCODE_DUMP_APP | 130 - 0x82 1000 0010 | Save - append |
| BINARY DATA: Following will cause no extension to be appended - command must use full filename |  |  |
| OPCODE_SAVE_DATC | 135 - 0x87 1000 0111 | Save - create (no overwrite) |
| OPCODE_SAVE_DATO | 136 - 0x88 1000 1000 | Save - create (overwrite) |
| OPCODE_SAVE_DATA | 137 - 0x89 1000 1001 | Save - append |
| EXECUTABLE FILES: Following save modes will cause ZolaDOS to append '.EXE' to the filename. |  |  |
| OPCODE_SAVE_CRT | 140 - 0x8A 1000 1010 | Save - create (no overwrite) |
| OPCODE_SAVE_OVR | 141 - 0x8B 1000 1011 | Save - create (overwrite) |
| OPCODE_SAVE_APP | 142 - 0x8C 1000 1100 | Save - append |

ERROR CODES

These are defined in:

- 6502 code: cfg_main.asm

- Go code: zdlib/zdconsts.go

| NAME | Value (dec) |
| --- | --- |
| ERR_FILE_READ | 7 |
| ERR_FILE_LIST | 13 |
| ERR_FILE_EXISTS | 22 |
| ERR_FILE_OPEN | 23 |
| ERR_FILE_DEL | 24 |
| ERR_FILENOTFOUND | 25 |
| ERR_FILE_BOUNDS | 26 |